1．Common functions

(initialcom)(/*[in]*/int port,/*[in]*/long baud,/*[out,retval]*/int *hdev)

Function

Initial the communication port.

Parameters

port: Serial port 1~20 when value 0~19. USB when value 100

baud: Baud rate (value: 9600~115200)

Return Value

Return handle value of reader (>0), negative value if failed.

Example

var icdev;

icdev=obj.initialcom (100,9600);//initial USB

//obj is the object of control , the same following

(exit)(/*[in]*/short icdev,/*[out,retval]*/short *resul)

Function

Close the communication port.

Parameters

icdev: the handle value of reader

Return Value

0 if success; Otherwise, NonZero

Example

obj.exit(icdev);

Remark

icdev must be released before next linking.

(hexTochar)(/*[in]*/BSTR hexstr,/*[in]*/short chlen,/*[out,retval]*/BSTR *charstr)

Function

Transform HEX string to corresponding ASC bytes array.

Parameters

hexstr: the string of HEX

chlen: the length of chars

charstr: the bytes array

Return Value

The bytes array.

Example

var hex=”30313233”;

var chstr=””;

chstr=obj.hexTochar(hex,4);

if(chstr!=”0123”)

alert(“error”);

(charToHex)(/*[in]*/BSTR charstr,/*[in]*/short hexlen,/*[out,retval]*/BSTR *hexstr)

Function

Transform ASC bytes array to corresponding HEX string.

Parameters

charstr: the bytes array.

hexlen: the length of HEX string.

hexstr: the string of HEX.

Return Value

The string of HEX.

Example

var chstr=”abcd”;

var hex=””;

hex=obj.charToHex(chstr,8);

if(hex!=”61626364”)

alert(“error”);

(GetDevSN)(/*[in]*/ short icdev, /*[out,retval]*/ BSTR* devSN);

Function

Get the device’s serial number

Parameters

icdev: the handle value of reader

Return Value

Device serial number as string

Example

var devSN= obj.GetDevSN(icdev);

2．Device Functions

(beep)(/*[in]*/short icdev,/*[in]*/short isec,/*[out,retval]*/short *resul)

Function

Make reader to beep.

Parameters

icdev: the handle value of reader

isec: time of beep.

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

st=obj.beep(icdev,10);             //beep 10ms

(srd_eeprom)(/*[in]*/short icdev, /*[in]*/ int offset, /*[in]*/ int length, /*[out,retval]*/BSTR* pData);

Function

Read the EEPROM data of reader

Parameters

icdev:	the handle value of reader

offset:	the start address to read

length:	the length to read

Return Value

The data of EEPROM as string

Example

var data;

var offset = 0;

var len = 5;

data = obj.srd_eeprom(icdev, offset, len);

(swr_eeprom)(/*[in]*/short icdev, /*[in]*/ int offset, /*[in]*/ int length, /*[in]*/BSTR strData,/*[out,retval]*/short* result);

Function

Write data to EEPROM of reader

Parameters

icdev:	the handle value of reader

offset:	the start address to read

length:	the length to read

strData:	the data ready write to EEPROM as string

Return Value

0 if success; Otherwise, Nonzero

Example

var data= “ATest”;

var offset = 0;

var len = 5;

var st;

st = obj.swr_eeprom(icdev, offset, len, data);

(lcd_setBright)(/*[in]*/short icdev, /*[in]*/ short fBright, /*[out,retval]*/short* result);

Function

Set the bright state of LCD

Parameters

icdev: the handle value of reader

fBright: values 0 to make LCD back light off, otherwise 1 to make on

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

st=obj.setBright(icdev, 0);             //make back light off

(lcd_dispstr_ex) (/*[in]*/short icdev, /*[in]*/ BSTR str, /*[in]*/ int line,/*[in]*/ int offset, /*[in]*/ int str_len, /*[in]*/ int flag,/*[out,retval]*/ short *result);

Function

Display string on the LCD

Parameters

icdev:	the handle value of reader

str:		the string for display

line:		line number the string show on

offset:	start address in the line

str_len:	the size of string,when flag values 0, size equals the char number of string, when flag values 1, size equals twice of char number

flag:		0 if show as ANSI string, or 1 for show as GB2312 string

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

var str = “123456”;

st=obj.lcd_dispstr_ex(icdev,str, 1, 0, str.length, 0);

(lcd_dispclear) (/*[in]*/ short icdev, /*[out, retval]*/ short* result);

Function

Clear string on the LCD

Parameters

icdev:	the handle value of reader

Return Value

0 if success; Otherwise, Nonzero.

Example

obj.lcd_dispclear(icdev);

(passInReady)  (/*[in]*/short icdev,/*[in]*/short waitTime,/*[out,retval]*/short* result);

Function

Set the device to state of ready for inputing

Parameters

icdev:		the handle value of reader

waitTime:	the time for inputing as second,device input will canceled if out of the time

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

st = obj.passInReady(icdev, 10);

(getPassPressed)  (/*[in]*/short icdev, /*[out, retval]*/ BSTR* strPass);

Function

Check the value user typed in

Parameters

icdev:		the handle value of reader

Return Value

String user typed in

Example

var data;

data = obj.getPassPressed(icdev);

(getPassConfirmed)  (/*[in]*/short icdev, /*[out,retval]*/ short* result);

Function

Get the input state

Parameters

icdev:		the handle value of reader

Return Value

00h if user typed the Enter button

A1h input overflow,25 chars max

A2h user typed the Cancel button

A3h the device in not in inputing state

A4h in the state of inputing

A5h input time out

Example

var st;

st = obj.getPassConfirmed(icdev);

switch(st)

{

case 0:altert(“You typed the enter button”);break;

case 0xA1:alter(“Password too long”);break;

case 0xA2:alert(“You cancel the input”);break;

case 0xA3:alert(“Keyboard is not in input mode”);break;

case 0xA4:alert(“Input running”);break;

case 0xA5:alert(“Input timeout”);break;

}

(passInEnd)  (/*[in]*/short hdev, /*[out,retval]*/ short* result);

Function

End the input, type in will invalid after this called

Parameters

icdev:		the handle value of reader

Return Value

0 if success; Otherwise, Nonzero.

Example

obj.passInEnd(icdev);

3．Class Mifare Card Functions

(reset)(/*[in]*/short icdev,/*[in]*/short Msec,/*[out,retval]*/short *resul)

Function

Reset the MCM (MIFARE read/write device core mode).

Parameters

icdev: the handle value of reader

Msec: the time to reset. Unit: 10ms

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

St=obj.reset(icdev,10);//RF reset100ms

(findcard)(/*[in]*/int hdev,/*[in]*/short mode,/*[out,retval]*/long *ncard)

Function

Find card,can return the card serial number in working area. (Contain the next functions: request, anticoll, select).

Parameters

hdev: the handle value of reader

mode: the mode of find card  (refer to appendix)

ncard: card serial number returned

Return Value

If success, returned the card serial number.

Example

var ncard=0;

ncard=obj.findcard(icdev,0);//mode 0 authentication

Remark

- In IDLE mode, after read-write operation, we use function halt() to end the card operation, only when the card out and re-enter the operating area, the reader can operate it once again.

- Related HEX function is findcardHex, recommend use findcardHex.

d

(findcardHex)(/*[in]*/int hdev,/*[in]*/short mode,/*[out,retval]*/BSTR *strcard)

Function

Find card,can return the card serial number in working area. (HEX string type).

Parameters

hdev: the handle value of reader

mode: the mode of find card  (refer to appendix)

strcard: card serial number returned

Return Value

If success, return the card serial number.

Example

var ncard=0;

ncard=obj.findcardHex(icdev,0);//mode 0 authentication

Remark

In IDLE mode, after read-write operation, we use function halt() to end the card operation, only when the card out and re-enter the operating area, the reader can operate it once again.

(request)(/*[in]*/short hdev,/*[in]*/short mode,/*[out,retval]*/short *resul);

Function

Find card request.

Parameters

hdev: the handle value of reader

mode: the mode of find card

Return Value

0 if success; Otherwise, Nonzero

Example

var st;

st = obj.request(icdev,0);

(anticoll)(/*[in]*/short icdev,/*[in]*/short mode,/*[out,retval]*/long *ncard)

Function

Preventing card conflict, return the card serial number.

Parameters

hdev: the handle value of reader

mode: the mode of find card

ncard: the card serial number returned.

Return Value

If success, return the card serial number.

Example

var ncard =0;

ncard=obj.anticoll(icdev,0);

Remark

You should call anticoll just after request, unless known the card serial number.

(select)(/*[in]*/short icdev,/*[in]*/long _Snr,/*[out,retval]*/short *resul)

Function

Select one card with special number from several cards.

Parameters

hdev: the handle value of reader

_Snr: the card serial number.

Return Value

0 if success; Otherwise, NonZero

Example

var st;

var snr=0;

st=obj.request(icdev,0);

snr=obj.anticoll(icdev,0);

st=obj.select(icdev,snr);

(loadkey)(/*[in]*/short hdev,/*[in]*/short mode,/*[in]*/ short sNr,/*[in]*/BSTR  key,/*[out,retval]*/short *resul)

Function

Load keys to RAM of Reader.

Parameters

hdev: the handle value of reader

mode: the mode of key verify.

Value as following:

For each sector of M1 card, there are three sets of corresponding password (KEYSET0 , KEYSET1, KEYSET2) in the reader, each password include keyA  and keyB , a total of six passwords, use 0~2, 4~6 to represent the six selections:

- KEYSET0 of KEYA

- KEYSET1 of KEYA

- KEYSET2 of KEYA

- KEYSET0 of KEYB

- KEYSET1 of KEYB

- KEYSET2 of KEYB

sNr: the sector number.

key: sector key  write to the reader

Return Value

0 if success; Otherwise, Nonzero

Example

var st;

/* load sector 0 key with set 0 and keyA*/

st = obj.loadkey(icdev,0,0,"FFFFFFFFFFFF");

(authentication)(/*[in]*/ short hdev,/*[in]*/short mode,/*[in]*/short snr,/*[out,retval]*/short *resul);

Function

Verify key.

Parameters

hdev: the handle value of reader

mode: the mode of key verify.

Value as following:

For each sector of M1 card, there are three sets of corresponding password (KEYSET0 , KEYSET1, KEYSET2) in the reader, each password include keyA  and keyB , a total of six passwords, use 0~2, 4~6 to represent the six selections:

- KEYSET0 of KEYA

- KEYSET1 of KEYA

- KEYSET2 of KEYA

- KEYSET0 of KEYB

- KEYSET1 of KEYB

- KEYSET2 of KEYB

snr: the sector number.

key: sector key  write to the reader

Return Value

0 if success; Otherwise, NonZero

Example

//authentication sector 4 with mode 0

var st;

st=obj.authentication(icdev,0,4);

if(st!=0)

{

alert("Authentication error!");

}

(read)(/*[in]*/short hdev,/*[in]*/short blk,/*[out,retval]*/BSTR *blkdata);

Function

Read content of card, for M1 card ,read one block data (16 bytes) once.

Parameters

hdev: the handle value of reader

blk: Address of block (M1- 0~63, MS70 – 0~255)

blkdata: [out] data of card.

Return Value

The data of block.

Example

var data=””;

data=obj.read(icdev,4);  //read data of block 4

(write)(/*[in]*/short hdev,/*[in]*/short blk,/*[in]*/BSTR wtdata,/*[out,retval]*/short *resul)

Function

Write data to card (HEX string). One block once (16 bytes).

Parameters

hdev: the handle value of reader

blk: Address of block (M1- 1~63, MS70 – 1~255)

wtdata: Data write to the card.

Return Value

0 if success; Otherwise, Nonzero

Example

var st;

st=obj.write(icdev,2,"11223344556677889900AABBCCDDEEFF");//write block 2

(directRead)(/*[in]*/short hdev,/*[in]*/short blk,/*[out,retval]*/BSTR *blkdata)

Function

Read data from card directly.

Parameters

hdev: the handle value of reader

blk: Address of block (M1- 0~63, MS70 – 0~255)

blkdata: Data read from card block.

Return Value

Data of card.

Example

var data=””;

data=obj. directRead (icdev,4);  //read block 4

(directWrite)(/*[in]*/short hdev,/*[in]*/short blk,/*[in]*/BSTR wtdata,/*[out,retval]*/short *resul)

Function

Write data to card directly (as ASC bytes).

Parameters

hdev: the handle value of reader

blk: Address of block (M1- 1~63, MS70 – 1~255)

wtdata: Data write to card block.

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

st=obj.directWrite(icdev,5,”13766668888”); /*write block 5*/

(halt)(/*[in]*/short hdev, /*[out,retval]*/short *resul)

Function

Abort operation of card.

Parameters

hdev: the handle value of reader

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

st=obj.halt(icdev);

Remark

When using findcard(), there is a parameter mode, card been set to HALT status when it is set value as 0, the card should be take out off operating area and put it again.

(changkey)(/*[in]*/short hdev,/*[in]*/ short secNr,/*[in]*/ BSTR keya,/*[in]*/ BSTR ctrlword,/*[in]*/short bk,/*[in]*/ BSTR keyb,/*[out,retval]*/

Function

Update key of special sector.

Parameters

hdev: the handle value of reader.

secNr: sector number.

keya: keyA to update.

ctrlword: control word to update.

bk:  reserved, set to 0.

keyb: keyB to update.

Return Value

0 if success; Otherwise, Nonzero.

Example

//update key of sector 3

var st;

st=obj. changkey(icdev,3,"FFFFFFFFFFFF","FF078069",0,"FFFFFFFFFFFF");

Remark

The parameters keya, ctrlword, and keyb, type as HEX string.

(initialval)(/*[in]*/short hdev, /*[in]*/short blk, /*[in]*/long value, /*[out,retval]*/short *resul)

Function

Initial the value of special block.

Parameters

hdev: the handle value of reader

blk: address of block

value: the value to initial block

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

var value=1000;

st=obj.initialval(icdev,1,value);        //initial the block 1 to 1000

Remark

Should call initial value before increment decrement value.

(increment)(/*[in]*/short hdev,/*[in]*/ short blk, /*[in]*/long value,/*[out,retval]*/short *resul);

Function

Increment value of special block.

Parameters

hdev: the handle value of reader

blk: address of block

value: the value to increment.

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

var value=10;

st=obj.increment(icdev,1,value); //increment 10 to block 1

(readval)(/*[in]*/short hdev,/*[in]*/ short blk, /*[out,retval]*/long *value)

Function

Read value of special block.

Parameters

hdev: the handle value of reader

blk: address of block

value: the value to read.

Return Value

The value of special block.

Example

var st;

var value;

value=obj.readval(icdev,1);       //read value of block 1

(decrement)(/*[in]*/short hdev,/*[in]*/ short blk,/*[in]*/ long value,/*[out,retval]*/ short *resul)

Function

Decrement value of special block.

Parameters

hdev: the handle value of reader

blk: address of block

value: the value to decrement.

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

var value=10;

st=obj.decrement(icdev,1,value);     //decrement the value of block 1

Remark

The parameters keya, ctrlword, and keyb, type as HEX string.

(transfer)(/*[in]*/short hdev,/*[in]*/short blk,/*[out,retval]*/short *resul)

Function

Transfer data in register to EEPROM.

Parameters

hdev: the handle value of reader

blk: address of block

Return Value

0 if success; Otherwise, Nonzero.

Example

var st;

st=obj.transfer(icdev,2);

Remark

This function should be called just after increment or decrement.

4．Contactless-CPU functions

(fcpureset)(/*[in]*/short icdev,/*[out]*/short *rlen,/*[out,retval]*/BSTR* rbuf)

Function

Power on and reset of CPU card.

Parameters

icdev: the handle value of reader

rlen: the length of returned information.

rbuf: the information returned.

Return Value

Information returned.

Example

var data=” “;

Var rlen;

data=obj.fcpureset(icdev,rlen);

Remark

This function should be called just after increment or decrement.

(fcpuCommandlink)(/*[in]*/short icdev,/*[in]*/short slen,/*[in]*/BSTR sdata,/*[in]*/short tt,/*[in]*/short FG,/*[out,retval]*/BSTR * rdata)

Function

APDU data exchange function.

Parameters

icdev: the handle value of reader

slen: the length of information to send.

sdata: command message to send.

tt:  delay time. unit: 10 ms

FG: split length, recommend small than 64.

rbuf: the information returned.

Return Value

Information returned.

Example

var cmd= "00A404000752736120417070";//12 bytes (HEX string)

Var data=” “;

data=obj.fcpuCommandlink(icdev,12,cmd,7,60);//delay 70ms,60 bytes once send.

Appendix

Mode of find card

There are three cases: IDLE mode, ALL mode, and special card mode.

0— means IDLE mode, one card once operate.

1— means ALL mode, multi card once operate.